Automatic Toll Collection System

1. Introduction

An Automatic Toll Collection System (ATCS) eliminates manual toll booths by using RFID technology. Vehicles are equipped with RFID tags, and toll is deducted automatically when they pass through the toll gate.

This project uses:

- ESP32 for processing & WiFi communication

- RFID module (MFRC522) for vehicle identification

- Firebase for real-time database & cloud storage

2. Objective

- Automate toll collection

- Reduce traffic congestion

- Maintain real-time vehicle records

- Enable cashless transactions

3. Working Principle

- Vehicle approaches toll gate

- RFID tag is scanned using MFRC522

- ESP32 reads UID of the tag

- UID is sent to Firebase

- Firebase checks balance

- If balance sufficient:

  - Deduct toll amount

  - Gate opens

- If insufficient:

  - Gate remains closed

  - Alert generated

4. Components Required

- ESP32

- RFID Module (MFRC522)

- RFID Tags/Cards

- Servo Motor (for gate control)

- Buzzer (optional)

- LEDs (status indication)

- Jumper wires

- Power supply

5. Firebase Setup

Use:

- Firebase Realtime Database

Sample Database Structure:

{
  "users": {
    "UID12345": {
      "name": "Rahul",
      "balance": 150
    }
  }
}

6. ESP32 Code

#include <WiFi.h>

#include <Firebase_ESP_Client.h>

#include <SPI.h>

#include <MFRC522.h>

#include <NTPClient.h>

#include <WiFiUdp.h>

// Wi-Fi credentials

const char* ssid = "narzo 50A";

const char* password = "12345678";

// Firebase credentials

#define API_KEY "AIzaSyAeJlnM73Y5b66FqmxB5Q4OggByzJ5gSHk"

#define DATABASE_URL "https://attendence-reader-default-rtdb.firebaseio.com/"

// Firebase config

FirebaseData fbdo;

FirebaseAuth auth;

FirebaseConfig config;

#define SS_PIN 5

#define RST_PIN 22

MFRC522 rfid(SS_PIN, RST_PIN);

// NTP-Time

WiFiUDP ntpUDP;

NTPClient timeClient(ntpUDP, "pool.ntp.org", 19800); // India = GMT+5:30

// UID-to-name mapping

String getNameFromUID(byte *uid) {

String id = "";

for (byte i = 0; i < 4; i++) id += String(uid[i], HEX);

if (id == "a1b2c3d4") return "Rahul";

else if (id == "12345678") return "Anjali";

return "Unknown";

}

void setup() {

Serial.begin(115200);

SPI.begin();

rfid.PCD_Init();

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) delay(1000);

config.api_key = API_KEY;

config.database_url = DATABASE_URL;

Firebase.begin(&config, nullptr);

timeClient.begin();

Serial.printf("Set bool... %s\n", Firebase.setBool(fbdo, F("/test/bool"), count % 2 == 0) ? "ok" : fbdo.errorReason().c_str());

}

void loop() {

if (!rfid.PICC_IsNewCardPresent() || !rfid.PICC_ReadCardSerial()) return;

String name = getNameFromUID(rfid.uid.uidByte);

timeClient.update();

String timestamp = timeClient.getFormattedTime();

String path = "/attendance/" + name;

Firebase.RTDB.pushString(&fbdo, path, timestamp);

Serial.println("Attendance marked for: " + name + " at " + timestamp);

delay(2000);

}

7. Features

- Real-time cloud database

- Cashless toll system

- Fast vehicle processing

- Scalable system

- Low-cost implementation

8. Limitations

- Requires internet connectivity

- RFID range is limited

- Security concerns (basic RFID tags)

9. Future Enhancements

- Mobile app integration

- Number plate recognition (AI + camera)

- Automatic recharge system

- SMS alerts

- Multi-lane toll management

10. Applications

- Highways toll booths

- Parking systems

- Smart city infrastructure

- Access control systems

11. Conclusion

This system demonstrates how IoT and cloud computing can automate traditional toll collection. It reduces human effort, minimizes delays, and ensures efficient transaction handling.